# 2.	FLYTHRU M&S REQUIREMENTS AND ACCEPTABILITY CRITERIA

The FLYTHRU M&S requirements have been developed in a hierarchy of requirements where the top-level requirement is the functionality that FLYTHRU must perform along with low-level requirements support that top-level requirement. The low-level requirements will be the functionality that will be verified but with a well formulated hierarchy of requirements, the top level requirement will be verified through its subordinate requirements.

## 2.1	Top-Level Requirement

At the top level, FLYTHRU is required to calculate prompt and delayed nuclear radiation environments from a threat device onto a receiver weapon for hostile and fratricide scenarios. FLYTHRU is required to calculate all prompt environments provided by GENFIT but calculate those points along an RB trajectory.  FLYTHRU is also required to calculate delayed radiation environments of an RB flying through or near a debris cloud of a previous weapon detonation.

This top-level requirement is expanded into a hierarchy of mid-level requirements below.  Acceptability criterion will be given for each requirement as applicable.

## 2.2	Requirements

Mid-level requirements support the objectives in the top-level and mid-level requirements but should be specific enough to be implemented and tested. The requirements are subdivided into five categories based on the nature of the requirement:

- Calculation Modes and User Options – Any requirement that defines the inputs and calculation modes of the software.

- Environments – Any requirement that defines radiation environments and responses to be calculated by FLYTHRU.

- Calculation Methodology and Accuracy – Requirements on the accuracy of the calculation. May specify the algorithm to be implemented if that is a specific.

- User Interface – Requirements related to the format of the inputs, outputs, and steps required for the user to execute a FLYTHRU simulation and view the results.

- Security – Requirements related to the handling of classified data.

- Software Quality – Requirements related to software standards or best practices for the design and implementation. The requirements should not dictate the design but should set standards to follow in the design phase.

### 2.2.1 Requirements for Calculation Modes (CM-*)

- FLYTHRU shall calculate the prompt nuclear environments from a threat weapon to a receiver in “Environment versus Altitude” (ICON(1) = 5) run mode.

  - Calculate prompt environment(s) at a follower RB location relative to a threat weapon intercepting a leading RB. The follower RB is on the same trajectory as the leading RB with a location offset defined by a time spacing, cross-range offset and up/downrange offset.

  - Calculate prompt environment(s) at a follower RB location as a function of intercept altitude of the leading RB. The user may specify starting and ending altitudes and number of calculation points. The user could alternatively specify starting and ending times relative to RB atmospheric reentry.

  - FLYTHRU shall calculate all prompt nuclear environments available in GENFIT/RES and specified in Section 2.2.2.

- FLYTHRU shall calculate the delayed nuclear environments from a threat weapon to a receiver in “Dose Rate Integral” (ICON(1)=0) mode.  The follower RB is on the same trajectory as the leading RB with a location offset defined by a time spacing, cross-range offset and up/downrange offset.

  - Calculate delayed gamma dose rate along a follower RB trajectory relative to a threat weapon intercepting a leading RB.

  - Calculate delayed gamma cumulative dose as a function of time from the hostile burst.

### 2.2.2 Requirements for Environments (ENV-*)

Note: Requirements unchanged from GENFIT/RES are listed in grey text.

- FLYTHRU shall calculate the radiation environments at the receiver weapon based on the selection of a threat weapon, threat weapon yield, dose type (IDOSKND), and response function (IKNDENV) that are defined in FLYTHRU.

  - Calculate dose from prompt radiation. Calculate from any combination of neutrons, secondary gammas, and gammas. (IDOSKND = 1)

    - Calculate neutron fluence (neutrons/cm2) (IKNDENV = 1)

    - Calculate energy fluence (MeV/cm2) (IKNDENV = 2)

    - Calculate 1 MeV equivalent fluence (neutrons/cm2) (IKNDENV = 9)

  - Calculate dose from X-rays. Calculate forward energy or total energy. (cal/cm2) (IDOSKND = 2)

  - Calculate dose rate from prompt radiation. Calculate from any combination of neutrons, secondary gammas, and gammas. (IDOSKND = 1)

    - Calculate neutron fluence rate (neutrons/cm2/s) (IKNDENV = 1)

    - Calculate energy fluence rate (MeV/cm2/s) (IKNDENV = 2)

    - Calculate 1 MeV equivalent fluence rate (neutrons/cm2/s) (IKNDENV = 9)

  - Calculate dose rate from X-rays. Calculate forward energy or total energy. (cal/cm2/s) (IDOSKND = 4)

  - Calculate ionizing dose in silicon from prompt radiation. Calculate from any combination of neutrons, secondary gammas, and gammas. (Rad-Si) (IDOSKND = 1) (IKNDENV = 4)

  - Calculate ionizing dose in silicon from X-rays. (Rad-Si) (IDOSKND = 2) (IKNDENV = 4)

  - Calculate ionizing dose in silicon from X-rays and all prompt radiation (neutrons, secondary gamma, & gamma). (Rad-Si) (IDOSKND = -2) (IKNDENV = 4)

  - Calculate ionizing dose rate in silicon from prompt radiation. Calculate from primary gamma. (Rad-Si/s) (IDOSKND = 1) (IKNDENV = 5)

  - Calculate ionizing dose rate in silicon from X-rays. (Rad-Si/s) (IDOSKND = 4) (IKNDENV = 5)

  - Calculate ionizing dose rate in silicon from X-rays and prompt radiation (primary gamma). (Rad-Si/s) (IDOSKND = -4) (IKNDENV = 5)

  - Calculate ionizing dose rate slope (ionizing dose double dot) from prompt radiation. Calculate from primary gamma. (Rad-Si/s2) (IDOSKND = 1)

  - Calculate ionizing dose rate slope (ionizing dose double dot) from X-rays. (Rad-Si/s2) (IDOSKND = 4)

  - Calculate ionizing dose rate slope (ionizing dose double dot) from X-rays and prompt radiation (primary gamma). (Rad-Si/s2) (IDOSKND = -11)

  - Calculate the peak neutron flux. (IDOSKND = 15) (neutrons/cm2/s)

  - Calculate the peak 1-MeV equivalent neutron flux. (neutrons/cm2/s) (IDOSKND = 16)

  - Calculate sustained Gamma Rate from Debris when running in “Dose Rate Integral” Mode (IDOSKND = 8)

  - Calculate sustained Neutron Rate from Debris when running in “Dose Rate Integral” Mode.

- FLYTHRU shall perform all prompt calculations for threat weapons defined in FLYTHRU by using the normalized environment data stored in the dose.dat, fdose.dat, xdose.dat, and xddose.dat files.

  - Calculate environments for prompt environments from neutrons, secondary gammas, and prompt gammas for pre-calculated weapons in dose.dat

  - Calculate environments for fission heating for pre-calculated weapons in fdose.dat.

  - Calculate environments for total x-ray dose for pre-calculated weapons in xdose.dat

  - Calculate environments for total x-ray dose rate for pre-calculated weapons in xddose.dat

- FLYTHRU shall calculate X-ray environments considering x-ray shielding from a database of materials (ISHIELD).

- FLYTHRU shall calculate time spacing at re-entry based on a user input x-ray fluence level.

  - FLYTHRU shall calculate time spacing based on a threat device and yield that may be different than the device used for prompt or delayed environment calculations.

- FLYTHRU shall find up/down range offset and/or cross range offset where the cumulative delayed dose is equal to a user input value.

### 2.2.3 Requirements for Calculation Algorithm and Accuracy (CA-*)

- FLYTHRU shall calculate the environments to the accuracy of the data files employing best practices interpolation.

- FLYTHRU shall reproduce legacy FLYTHRU results for all equivalent weapon, environment, and response function inputs.

- FLYTHRU shall calculate the threat environment based on the specifics of the threat weapon

  - Device name and number related to device in the FLYTHRU data files.

  - Device yield.

- Calculate integrated material depth (RHOR) along path using standard atmospheric density as a function of altitude.

- FLYTHRU shall perform all delayed calculations using best available delayed radiation modeling.

  - FLYTHRU shall use the low/high altitude RANC methodology.

  - FLYTHRU shall use the evacuated fireball geometry option in RANC.

  - FLYTHRU shall use the fireball rise option in RANC.

  - FLYTHRU shall use standard RANC fireball geometry (sphere, ellipsoid, and torus).

  - FLYTHRU shall calculate delayed gamma dose in Rad-Si and fluence in photons/cm2.

  - FLYTHRU shall calculate delayed neutron dose in Rad-Si and fluence in neutrons/cm2.

- FLYTHRU shall use table-based trajectories so that FLYTHRU can be run with unclassified notional trajectory inputs or classified real trajectories.

### 2.2.4 Requirements for User Interface (UI-*)

- FLYTHRU shall output equivalent data as legacy FLYTHRU for verification purposes.

- FLYTHRU shall be available as a stand-alone calculation library (DLL) or console executable so runs can be performed independent of any GUI.

- A GUI shall be developed that guides a user through the assembly of a FLYTHRU input file.

- The GUI or a utility script shall be developed to automatically plot the output from FLYTHRU.

- FLYTHRU shall output results into a formatted human readable ASCII file.

- FLYTHRU inputs should be as straight-forward as possible using names and descriptive keywords instead of numbers when possible.

- FLYTHRU GUI shall alert the user with a when a weapon, environment, and response selection is invalid.

### 2.2.5 Requirements for Security (SEC-*)

- FLYTHRU source code shall not contain any classified data. All classified data shall be contained in data files that can be clearly labeled.

- FLYTHRU shall allow the user to input the classification of the results. FLYTHRU will appropriately label the output with that classification.

### 2.2.6 Requirements for Software Quality (SQ-*)

- FLYTHRU code shall be structured into easy to understand functions and data will be encapsulated into logical data structures.

- FLYTHRU source code shall include verbose comments describing inputs and outputs of every function and descriptions of calculations in each section of the code.

- FLYTHRU shall be thread-safe such that it will operate correctly if called in parallel.

- FLYTHRU shall have a User’s manual describing all input options and output data.

- FLYTHRU shall have a Programmers manual describing the internal data representation and calculation steps.

- All source code, data files, and documents shall be maintained on a configuration management system.

## Verification Requirements

Each Low-level requirement must be verifiable based on the following four methods. Each requirement should identify how that requirement can be verified during verification and validation. Each requirement may be verifiable through multiple methods.

- Demonstration (D) - The operation of the system that relies on observable functional operation not requiring the use of special test equipment or subsequent analysis.

- Test (T) - The operation of the system using special test equipment to collect data for later analysis.

- Analysis (A) - The processing of accumulated data obtained from other qualification methods. Examples are reduction, interpretation, or extrapolation of test results.

- Inspection (I)- The visual examination of the software, documentation, etc.